Visual SLAM

Objective:

Solve the problem according to the requirements below, think critically about the scenarios on visual SLAM.

Requirements:

We want to achieve the level of autonomy with No-GPS in situations such as:

1. Drone flying within 1km range and coming back to the initial position.

2. If drone flies over range of 10km, render some concept architecture to use algorithms or ideas to achieve this.

3. If drone completely looses its control on positioning and navigation, any ways to bring it back (clue: while flying forward it might have collected some impressions/images/landmarks)

Solution:

Conceptual solution for achieving No-GPS autonomy in drone navigation using Visual SLAM:

1. Short-Range Autonomy (≤ 1 km)

- Approach: Use Visual SLAM with onboard cameras and IMU (Inertial Measurement Unit).

- Method:

- Real-time feature extraction (ORB, SIFT, or FAST features).

- Build a local map of landmarks (trees, buildings, terrain textures).

- Fuse IMU data with visual odometry for drift correction.

- Outcome: Drone can fly within 1 km and reliably return to its starting point by retracing the mapped trajectory.

2. Extended Range Autonomy (~10 km)

- Challenge: Visual SLAM alone accumulates drift over long distances.

- Concept Architecture:

  - Hierarchical Mapping: Divide the environment into sub-maps (local SLAM maps) and stitch them into a global map.

  - Loop Closure Detection: Use place recognition (e.g., Bag of Visual Words, deep learning-based image retrieval) to detect revisited areas and correct accumulated drift.

  - Multi-Sensor Fusion: Integrate:

    - Visual SLAM (cameras)

    - IMU (motion stability)

    - Magnetometer (heading reference)

    - Optional LiDAR or depth sensors (for robust mapping in texture-poor areas).

  - Cloud/Edge Assistance: Periodically offload map fragments to a ground station or edge server for optimization and drift correction.

- Outcome: Enables reliable navigation over 10 km without GPS, maintaining global consistency.

3. Recovery from Complete Positioning Loss

- Scenario: Drone loses track of its position mid-flight.

- Recovery Strategy:

  - Landmark Recall: Use stored visual impressions (images/landmarks) from earlier flight to re-localize when similar scenes are encountered.

  - Image-Based Localization: Match current camera feed with previously mapped features to estimate position.

  - Fallback Mode: If re-localization fails:

    - Drone switches to “safe return” mode, flying back along the last known trajectory using dead-reckoning with IMU.

    - If still unsuccessful, drone hovers and transmits captured images to ground station for operator-assisted re-localization.

- Outcome: Drone can regain orientation and attempt return even after losing navigation control.

Block Diagram:

Mission Control

- RRT*/ A* Path Planning

- Obstacle Avoidance

- Return-to-Home Logic

Communication

- Map Offloading

- Edge Optimization

- Operator Assist

Recovery System

- Bag of Visual Words

- Deep Image Retrieval

- Dead-Reckoning (IMU)

Sensor Fusion

- EKF/ Graph Optimization

- VIO (MSCKF)

- Drift Correction

SLAM Engine

- ORB-SLAM3 (features)

- Visual Odometry

- Map Building (Graph)

- Loop Closure (DBoW2)

Sensor Layer

- Camera (RGB/D)

- IMU (Accel/Gyro)

- Magnetometer

- LiDAR/Depth (Optional)

Explanation of Flow:

- Sensor Layer

  - In Sensor Layer, Cameras + IMU provide raw data.

  - An optional LiDAR/Depth improves robustness in texture-poor environments.

- SLAM Engine

  - It extracts features, builds maps, and performs loop closure to reduce drift.

  - In hierarchical mapping, SLAM engine ensures scalability for long-range missions (~10 km).

- Sensor Fusion

  - It combines visual odometry with IMU, magnetometer, and other sensors using Extended Kalman Filter (EKF) or graph optimization.

  - It corrects drift and stabilizes navigation.

- Recovery System

  - It uses stored landmarks and images for re-localization if navigation is lost.

  - In ‘safe return mode’, the recovery system retraces last known trajectory or hovers for operator assistance.

- Communication Layer

  - It offloads map fragments to edge/cloud for optimization.

  - It provides operator with visual data for assisted recovery.

- Mission Control

  - It handles path planning, obstacle avoidance, and return-to-home logic.

  - It ensures autonomy even in GPS-denied environments.

Algorithm Annotations:

- SLAM Engine

  - ORB-SLAM3: Robust visual SLAM framework for monocular, stereo, and RGB-D cameras.

  - DBoW2 (Bag of Words): Used for loop closure detection and place recognition.

  - Graph Optimization (g2o, Ceres Solver): Ensures global map consistency.

- Sensor Fusion

  - EKF (Extended Kalman Filter): Combines IMU + visual odometry for smoother state estimation.

  - MSCKF (Multi-State Constraint Kalman Filter): Efficient visual-inertial odometry.

  - Drift Correction: Periodic re-localization using landmarks.

- Recovery System

  - Bag of Visual Words / Deep Image Retrieval (NetVLAD, SuperGlue): Matches current scene with stored impressions.

  - Dead-Reckoning (IMU fallback): Retraces last known trajectory when vision fails.

- Mission Control

  - Path Planning: Algorithms like RRT* (Rapidly-exploring Random Tree) or A* for efficient navigation.

  - Obstacle Avoidance: Real-time detection using depth sensors or stereo vision.

  - Return-to-Home Logic: Uses re-localized map or IMU fallback to guide drone back.

- Communication Layer

  - Edge/Cloud Optimization: Offload map fragments for bundle adjustment and drift minimization.

  - Operator Assist: Human-in-the-loop re-localization if autonomous recovery fails.

Phased Roadmap for No-GPS Drone Autonomy (Visual SLAM):

Phase 1: Short-Range Prototype (≤ 1 km)

- Goal: Demonstrate reliable return-to-home without GPS.

- Key Components:

  - ORB-SLAM3 for monocular/stereo camera input.

  - IMU fusion via EKF for drift correction.

  - Basic path retracing logic.

- Milestone: Drone can fly within 1 km, build a local map, and autonomously return to its starting point.

Phase 2: Extended Range Autonomy (~10 km)

- Goal: Scale SLAM to longer missions with minimal drift.

- Key Components:

  - Hierarchical mapping (divide into sub-maps).

  - Loop closure detection using DBoW2 or NetVLAD.

  - Multi-sensor fusion (IMU + magnetometer + optional LiDAR).

  - Graph optimization (g2o/Ceres Solver) for global consistency.

- Milestone: Drone can fly ~10 km, maintain a coherent global map, and navigate back reliably.

Phase 3: Recovery & Robustness

- Goal: Ensure resilience when navigation is lost.

- Key Components:

  - Landmark recall via Bag of Visual Words or deep image retrieval (SuperGlue, NetVLAD).

  - Dead-reckoning fallback using IMU trajectory.

  - Safe return mode (hover + operator assist if re-localization fails).

- Milestone: Drone can autonomously attempt recovery and return even after losing positional awareness.

Phase 4: Cloud/Edge Integration

- Goal: Enhance scalability and reduce onboard computation.

- Key Components:

  - Offload map fragments to edge/cloud for bundle adjustment.

  - Operator-assisted re-localization using transmitted images.

- Milestone: Drone achieves long-range missions with reduced onboard load and human-in-the-loop safety net.

Phase 5: Full Autonomy & Mission Control

- Goal: Enable complex missions in GPS-denied environments.

- Key Components:

  - Path planning (RRT, A).

  - Obstacle avoidance using stereo vision or depth sensors.

  - Return-to-home logic integrated with SLAM + recovery system.

- Milestone: Drone can autonomously plan, execute, and recover from missions without GPS.

Roadmap Timeline: No-GPS Drone Autonomy (Visual SLAM):

Phase 1 (Prototype, ≤1 km)

---------------------------------

- ORB-SLAM3 + IMU fusion (EKF)

- Local map building

- Path retracing logic

Milestone: Reliable short-range return

Phase 2 (Extended Range, ~10 km)

---------------------------------

- Hierarchical mapping (sub-maps)

- Loop closure (DBoW2/NetVLAD)

- Multi-sensor fusion (IMU, magnetometer, LiDAR)

- Graph optimization (g2o/Ceres)

Milestone: Long-range navigation with minimal drift

Phase 3 (Recovery & Robustness)

---------------------------------

- Landmark recall (Bag of Visual Words, SuperGlue)

- Image re-localization

- Dead-reckoning fallback (IMU trajectory)

- Safe return mode

Milestone: Autonomous recovery after navigation loss

Phase 4 (Cloud/Edge Integration)

---------------------------------

- Map offloading to edge/cloud

- Bundle adjustment optimization

- Operator-assisted re-localization

Milestone: Scalable missions with reduced onboard load

Phase 5 (Full Autonomy & Mission Control)

---------------------------------

- Path planning (RRT*, A*)

- Obstacle avoidance (stereo/depth sensors)

- Return-to-home logic

Milestone: Complex GPS-denied missions fully autonomous

Architecture Overview:

Drone Autonomy System

--------------------------------------------------------------------------------------------------------------------------

Sensors: Cameras, IMU, Magnetometer, LiDAR (opt)

SLAM Engine: ORB-SLAM3, Visual Odometry, Loop Closure

Sensor Fusion: EKF/MSCKF, Drift Correction

Recovery: Landmark Recall, Image Re-localization

Mission Control: Path Planning (RRT*/A*), Obstacle

Avoidance, Return-to-Home Logic

Communication: Edge/Cloud Map Offloading, Operator Assist

Phased Roadmap:

- Phase 1 (Prototype ≤1 km): ORB-SLAM3 + IMU fusion, reliable short-range return.

- Phase 2 (~10 km): Hierarchical mapping, loop closure, multi-sensor fusion, drift minimization.

- Phase 3 (Recovery): Landmark recall, image re-localization, dead-reckoning fallback.

- Phase 4 (Cloud/Edge): Map offloading, bundle adjustment, operator-assisted recovery.

- Phase 5 (Full Autonomy): Path planning (RRT/A), obstacle avoidance, mission execution.